AppsGate Server Developer’s Companion

June 26th, 2015

Abstract

The AppsGate Server runs an HMI middleware to manage sensors and actuators in the home and enable End-User Programming.  This document is dedicated for developers to understand the software architecture of the HMI Middleware. The main purpose is mostly integration of new devices and services at home. It also acts as a reference document for the HMI framework and the corresponding API.

Keywords

Software Architecture, Specifications, API, middleware, Sensors & Actuators Integration, OSGi, ApAM, Fuchsia

Document History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| V1.0 | 24/05/2014 | Cedric Gérard | First Release (now obsolete) |
| V1.1 | 22/12/2014 | Thibaud Flury | New outline for the document (draft) |
| V1.12 | 09/01/2015 | Thibaud Flury | Draft for parts 1, 2, 3 |
| V1.2 | 26/06/2015 | Thibaud Flury | Refactoring of architecture |

Contributors

| Name | Email | Organization |
| --- | --- | --- |
| Cedric Gérard | cedric.gerard@carnot-lsi.com | Formerly UJF/LIG |
| Thibaud Flury | thibaud.flury@imag.fr | UJF/LIG |

Table of Contents

1	Use Cases and Requirements for the HMI Middleware	6

1.1	HMI Middleware Use Cases	6

1.1.1	Configuration	6

1.1.2	Control	6

1.1.3	End-User Programming	7

1.1.4	Understanding	7

1.2	Main Requirements for the HMI Middleware	7

1.2.1	The middleware is dynamic by nature	7

1.2.2	The middleware need loose coupling	8

1.2.3	Live extensibility of the middleware	8

1.2.4	Resilience	8

1.2.5	Persistency	8

1.2.6	The house should work even when end-users are not there	9

1.2.7	The middleware should be responsive	9

1.2.8	The middleware should respect privacy and enforce security	9

1.3	Deployment Constraints	9

1.3.1	Running on the Set-Top-Box	9

1.3.2	More than a Proof-Of-Concept	9

2	Functional Architecture	10

2.1	Core HMI Middleware	10

2.1.1	Adapters as a means to support technological integration	10

2.1.2	“Core world” as a uniform representation of the physical world and of the cloud digital world	11

2.1.3	Mechanisms as a way to support dynamicity automatically	11

2.2	Extended HMI Middleware	11

2.2.1	Context Manager	11

2.2.2	Trace Manager	12

2.2.3	The End-User Development Environment	12

2.2.4	Extended Devices and Services	12

2.3	Communication	12

3	Technology Choices and Deployment Architecture	13

3.1	Technology Baseline for the AppsGate Server	13

3.1.1	Java	13

3.1.2	JSON	13

3.1.3	OSGi	13

3.2	Communication between Client and Server	14

3.3	Data Persistence	14

3.4	Development Frameworks	15

3.4.1	ApAM	16

3.4.2	Fuchsia	18

3.5	Using the Project Source Code	19

3.5.1	Useful Links (source code, documentation, bug reports)	19

3.5.2	Requisite to build the project	19

3.5.3	Building and running the server with command line tools	19

3.5.4	Project hierarchy	20

4	Core HMI Middleware	21

4.1	Core HMI Middleware by the example: Adding a new Device Type	23

4.1.1	Process overview	23

4.1.2	Step 1: Development of a Core Device Specification	24

4.1.3	Step 2: Adding a Communication Technology Adapter	26

4.1.4	Step 3: Development of a Core Device Type Implementation	27

4.2	API Reference: Core HMI Uses Cases	29

4.2.1	A Home Device become Available or Unavailable	30

4.2.2	The AppsGate client can get the Devices List or get one Device Status	31

4.2.3	The AppsGate client can send commands or retrieves events from the Device	31

4.3	API Reference: The CoreObjectSpec Component	33

4.3.1	Core Object properties	34

4.3.2	CoreObjectSpec generic class interface	34

4.3.3	NotificationMsg generic message interface	35

4.3.4	Core Object Description (JSON)	35

4.3.5	Core Object Behavior (JSON)	36

4.3.6	Notification Message Description (JSON)	37

4.3.7	Helper Classes	37

4.3.8	Technology Adapters	39

5	Extended HMI Middleware	39

5.1	Extended HMI Uses Cases	39

5.2	API Reference: Context Management	41

5.2.1	API Reference: MongoDBConfiguration	42

5.2.2	Device Properties Manager Service	45

5.2.3	Context History Manager Service	46

5.2.4	Places Manager Service	46

5.2.5	Users Manager Service	46

5.3	End-User Programming	46

5.4	Other provided Extended Services	46

5.5	API: Guidelines for Extended Services Integration	46

6	Communication between CHMI, EHMI and AppsGate Clients	46

6.1	Communication Architecture	46

6.2	CHMI Proxy Service	48

6.2.1	CHMIProxySpec service interface	50

6.2.2	Generic Command	51

6.3	Client Communication Manager Service	52

6.4	EHMI Proxy Service	53

Annex A: ApAM components Application Architecture	55

Annex B: Existing Core Services and Devices Specifications	56

Existing Core Service/Device Type Codes	56

CoreSmartPlugSpec	57

CoreSwitchSensorSpec	57

CoreTempSensorSpec	57

CoreCO2SensorSpec	57

CoreContactSensorSpec	57

CoreKeyCardSensorSpec	57

CoreLumSensorSpec	57

CoreOccupancySpec	57

CoreOnOffActuatorSpec	57

CoreColorLightBulbSpec	57

CoreMailSpec	58

CoreMediaBrowserSpec	58

CoreMediaPlayerSpec	58

CoreMobileDeviceSpec	58

CoreShutterSpec	58

CoreDomiCubeSpec	58

CoreTVSpec	58

CoreBadgeDoorSpec	58

CoreCalendarSpec	58

CoreClockSpec	58

Annex C: Currently Supported Technology Adapters	59

Clock and Alarm Service	59

EnOcean Devices using Ubikit Adapter	59

Philips HUE Lamps	59

UPnP Media players and browsers	59

Domicube	59

Weather Web Service	59

Mail Web Service	59

Calendar Web Service	60

Pace TV Service	60

ARD alarm service	60

KNX Devices	60

Watteco Devices	60

# Use Cases and Requirements for the HMI Middleware

## HMI Middleware Use Cases

Paragraphe introductif:

Donner le pouvoir à l’utilisateur pour contrôler sa maison

lien avec le companion doc sur le côté client

The HMI Middleware must address four major uses cases. Configuration of the house and Control of the devices are the basics of any connected home. End-User Programming and Understanding the house behaviour are specifics to the project.

Figure 1. Main Use Cases for HMI Middleware

### Configuration

The end-user must be able to configure the house, the connected devices and the web services. The living areas should have a name and relations such as membership (the kitchen is on the ground floor, inside the home). Inhabitants of the home should also have names along with specific roles and privileges.  The connected devices should also have name, they might be located inside living areas. The installation and configuration process for devices ands services must be as seamless as possible, hiding and abstracting those issues of heterogeneous protocol such as pairing.

### Control

The end-user must be able to get the current state of its devices, including the functional aspects (the power consumption measured by a smart plug) and its non-functional aspects (such as battery level, radio range, failure report). If the device is an actuator (such as a dimmable light), the end-user must be able to send commands to the device (turn on the light, send a mail, play a music…).

### End-User Programming

This is the main focus of the project. Instead of using particular business application, the end user must be able to program the behaviour of the home.  Two development paradigms can be used.

- Rule-based programming (Event->Condition->Action, i.e: when temperature change, if measured temperature is less than 20°C, then turn on heat).

- Imperative programming defines sequences of commands (including expression evaluation and loops).

### Understanding

This use case is mandatory as the home can act on behalf of the end-user, especially with End-User Programming. As for computer engineering, the system behaviour is deterministic but sometime hardly understandable. Multiple events might occur at the same time and multiple programs might be started in parallel not to mention errors and bugs introduced by the end-user itself. Tools must be provided to follow what happened and why.

## Main Requirements for the HMI Middleware

Drawing from the previous use cases and a-priori knowledge of the business field of home automation we can establish the main requirements for the HMI Middleware. These requirements drive the fundamental architectural choices for the whole system.

Figure 2. HMI Middleware requirements

### The middleware is dynamic by nature

The set of home devices is not fixed; it depends of the user’s installation and is likely to evolve as time passes by. User may buy and install new equipment. A device can fail or runs short of battery. Furthermore, most home automation protocols are hardly reliable.

As a consequence, the devices (and by extension, any piece of software) may appear and disappear dynamically in an unpredictable manner.

Software modularity is not enough to address this requirement. We need a dynamic and component-based approach. Each device might be depicted by a particular software component.

### The middleware need loose coupling

On the other side the HMI Middleware is used jointly with AppsGate Clients. Those Clients are responsible to present users interfaces and are described in a separate document [TODO Ref].

Several users interface can be used at the same time (thin web-based, fat client application, smartphone application, even tangible interaction) and they can connect/disconnect with no warning.

Service-Oriented Architecture is an approach that solves the issue of loose coupling between the server and the client. Between the components in the server, it allows the software modules to be dynamically bound/unbound with no harm for the whole infrastructure. The middleware must support both Synchronous and Asynchronous communication between client and service.

### Live extensibility of the middleware

As stated previously, the set of home device may evolve as time passes by. But the technologies used will also change. For almost each technology, the system needs some sort of driver to exchange data between the HMI Middleware and the low-level layers of the corresponding home automation protocol. The system should not be stopped or worse rebuilt if a new protocol becomes the latest fashion.

The dynamic and component-based approach is also an answer to this issue. But particular components should be created to act as Technology Adapters. Each adapter acts as a Factory of component to discover and communicate with home devices relative to a particular technology.

### Resilience

Loose coupling and dynamicity can prevent the system from crashes and allows dynamic reconfiguration. But the HMI Middleware must maintain an acceptable level of service in the face of faults.

To benefit from this dynamic reconfiguration capability, software component should be expressed using high abstraction levels: by their Specification instead of their Implementation. Multiple implementation of the same component should be provided to allow the system to select the best one and bind it at runtime. The HMI Middleware architecture would then be described as composition of these specification components in a Declarative fashion.

### Persistency

The HMI Middleware, as a whole, is stateful application. The house is holding a particular configuration, with its living areas, its users, the configuration of devices, the end-user programs and their states.

Persistency should be achieved using a software layer responsible to interact with a database management system. To allow the “understanding” use case, the system should keep tracks of the states of sensors. This may represents a huge set of data to maintain. A trade-off between long-term persistency (i.e: configuration of the house) and short-term persistency (i.e: values measured by individual sensors) is mandatory.

### The house should work even when end-users are not there

The HMI Middleware must be always available, autonomic and running as a background service. End-user apparatus such as smartphone, desktop computers, laptop or tablet are not convenient target for the AppsGate Server because they may be turned off or being used outside the home. The requirements are a continuous electric power supply and networks interfaces.

The project focus on deployment on a Set-Top-Box, that provides these capabilities.

### The middleware should be responsive

The system needs to react with a short delay because any state change of a sensor can trigger an action of an actuator. And above all else, the HMI Middleware needs to maintain a unique and coherent state at anytime.

This enforces a centralized approach. The home devices might be distributed throughout the house but the end-user program must run in a single system.

### The middleware should respect privacy and enforce security

The HMI Middleware is collecting data about the house and its inhabitants.  This data is highly private and critical. The cloud is not secure (and not enough responsive).

Private data from the home should remain in the home. The AppsGate server must be located inside the end-user house, attached to the Local Area Network.

## Deployment Constraints

### Running on the Set-Top-Box

The very purpose of AppsGate is to enable new services of the home using the Set-Top-Box. The Set-Top-Box is an Application gateway able to run Android application.

As a consequence, the HMI Middleware must work on the Set-Top-Box developed by the project upon the Android operating system.

### More than a Proof-Of-Concept

To evaluate the relevance of end-user programming, the AppsGate Server will be used in field user tests. Real families, in their real homes, during everyday life, will use this system.

The system should be robust enough (from a usability principle), to be up and running and available at anytime during two weeks. The system must not disturb or disrupt their daily activities.

# Functional Architecture

The HMI Middleware can be divided according to three functional areas.

- The Core HMI Middleware is responsible to abstract Home Devices and Cloud Services into a uniform representation set.

- The Extended HMI Middleware holds the business part of the project, managing Context and End-User Programming.

- The Communication part allows interaction between CHMI and EHMI and also between the AppsGate Server and AppsGate Clients.

Figure 3. AppsGate Server Functional Architecture

## Core HMI Middleware

The CHMI middleware bridges the gap between the external world and AppsGate. Its functional coverage includes:

- Technological integration of an heterogeneous set of sensors/actuators network protocols such as EnOcean and UPnP

- Uniform representation and management of EnOcean/UPnP/etc. compliant physical devices as well as of third parties services, such as Google calendar available from the cloud.

- Automatic support for dynamicity.

### Adapters as a means to support technological integration

Devices from the Physical World are integrated by the way of Technology Adapters. There is one adapter per type of protocol supported by the CHMI middleware. For example, the EnOcean adapter and the UPnP adapter bridge the gap between the physical devices supported by the EnOcean and UPnP protocols respectively, and the Core World.

### “Core world” as a uniform representation of the physical world and of the cloud digital world

Core World denotes the representation that the core HMI middleware has about the physical world and of the cloud digital world. This representation is expressed in terms of Core Devices. CoreTemperatureSensor is a component that represents a physical temperature sensor handled by one or more physical sensors network protocols (e.g., the EnOcean sensor network protocol). Similarly, CoreCalendar is the abstract representation of a Calendar service made available in the cloud (e.g. Google Calendar for instance).

### Mechanisms as a way to support dynamicity automatically

A difficulty in developing applications that use and control the physical world is that this world is dynamic. Dynamic means that the entities of the physical world, thus their representation in the Core World, appear/change/disappear as a result of events that happen in the physical world. The middleware handle these events automatically such that the system is resilient to changes of the physical world.

## Extended HMI Middleware

The EHMI middleware is an extensible set of software components designed for facilitating the development of smart environments. For the purpose of the AppsGate project, it includes:

- A Context Manager

- A Trace Manager

- An End-User Development Environment

- Extended Devices and Services

### Context Manager

A proper context manager should cover the followings levels of abstraction:

- At the lowest level, the sensing layer corresponds to numeric observables produced by the Core World.

- The perception layer provides symbolic observables at the appropriate level of abstraction.

- The situation and context identification layer identifies the current situation and context from observables, and detects conditions for moving between situations and contexts. Services in this layer specify the appropriate entities, roles, and relations for operating within the user’s activities. They can be used to predict changes in situation or in context, and thus anticipate needs of various forms (system-centric needs as well as user-centric needs).

Actually, the Context Manager is much simpler; it is merely a persistency manager for the state and configuration for the home. The sensing layer is actually managed by the TraceManager.

### Trace Manager

The Trace Manager has been developed to record events and states about end-user programs and devices that will make it possible for end-users to analyze the behavior of their smart home and repair the causes of malfunctioning (if any) or unexpected behaviour.

AppsGate Debugger, the component used for visualizing and browsing event traces, runs on AppsGate clients.

### The End-User Development Environment

The current version of the End-User Development Environment includes an editor for editing programs using a pseudo-natural language, a program interpreter, as well as support for understanding and debugging the behavior of an AppsGate smart home. The editor runs on clients whereas the interpreter runs on the AppsGate server.

### Extended Devices and Services

Any CoreObject may be reused and enriched as an ExtendedObject in the Extended HMI Middleware. More generally, those services that are not atomic and/or canonical to be generic (the same function provided by multiple devices manufacturer or device provider) and reused by multiple applications should not be represented as Core Devices in the CHMI Middleware.

It includes but is not limited to:

- Domain specific applications such as security, heating, …

- Composition or aggregation of Core Devices

- Aggregation and Interpretation of Context data

- Helpers services for AppsGate (for instance to control the behavior of Technology Adapter, to configure a WebService, ….)

## Communication

The Communication functional area allows interaction between CHMI and EHMI and also between the AppsGate Server and AppsGate Clients. It includes:

- The CHMI Proxy to manage communication from/to Core Devices

- The EHMI Proxy to manage communication from/to Extended Devices and AppsGate business components

- The Client Communication component to manage communication from/to the AppsGate Clients

Each of these components supports both synchronous and asynchronous communication.

# Technology Choices and Deployment Architecture

## Technology Baseline for the AppsGate Server

All developments of the HMI Middleware rest on three choices that act as technology baseline:

- A programming language: Java

- A description language: JSON

- An execution framework to support a dynamic component model: OSGi

One strong constraint of the project is to run on a Set-Top-Box powered by Android. Android comes with a Development ‘s kit, a specific programming language and its own execution framework. But it doesn’t sound as good as it is. Android is designed to work on mobile device and therefore minimize energy consumption.

It is not a good pattern for server applications and a Set-Top-Box is always on. In addition, to perform field user tests, the HMI middleware needs to run also on a small computer.

### Java

Java has been selected as programming language to cope with these constraints. Only minor adaptations are required to work with the Android virtual machine (essentially replacing some dependencies and libraries missing, removing inner classes, etc.). The project is currently built against Java 6 and supports above versions.

It should support any implementation of the java compiler but it has been successfully tested upon the Oracle Java SE 6 implementation.

### JSON

Two main description language fulfill the requirements of exchanging data between the server and the client (being a web application): XML and JSON. The latter has been selected because it is lightweight and works well with Javascript (the technology baseline for the client).

### OSGi

The OSGi (Open Service Gateway initiative) specification describes a modular system and a service platform for the Java programming language that implements a complete and dynamic component model, something that does not exist in standalone Java/VM environments. Applications or components, coming in the form of bundles for deployment, can be remotely installed, started, stopped, updated, and uninstalled without requiring a reboot; management of Java packages/classes is specified in great detail. Application life cycle management is implemented via APIs that allow for remote downloading of management policies. The service registry allows bundles to detect the addition of new services, or the removal of services, and adapt accordingly.

[OSGi Alliance]

The deployment of software modules as OSGi bundle is very similar to Android and therefore facilitates the adaptation between the two paradigms. Apache Felix implements the OSGi R4 Specifications and runs on Android, it has been chosen as a basis for the project. The project currently uses Apache Felix Framework Distribution v4.2.1.

The OSGi framework is structured into the following functional layers:

Bundles. Bundles are jar components with extra manifest headers (i.e. kind of meta-descriptions including a human readable name, a unique identifier, a version number, the class name to be invoked once the bundle is activated, the java packages that the bundle provides to, and requires from, the external world).

Services.The services layer connects bundles dynamically by offering a publish-find-bind model for plain old Java Interfaces (POJI) or Plain Old Java Objects POJO.

Service Registry. The service registry layer provides API for service management (Service Registration, Service Tracker and Service Reference).

Life-Cycle. The Life-cycle layer provides API for the life cycle management of bundles (install, start, stop, update, and uninstall).

Modules. This layer defines encapsulation and declaration of dependencies (how a bundle imports and exports code).

Security. This cross layer handles the security aspects by limiting bundle functionality to pre-defined capabilities such as access to critical resources.

An AppsGate ecosystem such as a smart home, is a highly distributed dynamic environment. This means that services and processors appear and disappear frequently across a possibly unreliable set of networks and network protocols. Service continuity must be ensured as well as the management of the concurrent execution of numerous applications. OSGi is not sufficient to support all these requirements at runtime.

## Communication between Client and Server

The reference AppsGate Client has been designed as a Web Application using HTML, CSS and Javascript [TODO: ref client companion]. HTTP Requests are the common way for a web client to pull data from the server but it is a request-response mechanism, it does not enable asynchronous communication neither communication initiated by the server.

The AppsGate Client must be able to reacts to events in the server. The WebSocket protocol has been chosen to handle communications between the Server and the Client(s). It provide full-duplex communication channel over HTTP, allowing queries and events.

Through this communication channel, the server and the client exchange JSON documents, being queries (for a home device to execute a command) or messages (an event coming from a sensor).

## Data Persistence

Managing persistence of a Smart Home represents a huge set of data, with performance and flexibility issues. Classical relational databases are not sufficient to solve these issues.

NoSQL databases allow a separation of concerns of storing a lot of information (sensors might push a numerous states changes in a short time) and the use of these data. The data model is able to evolve and being enhanced though time.

MongoDB has been selected as a widespread document-oriented database structured with optimized JSON-like documents.

## Development Frameworks

ApAM (Application Abstract Machine) is a component-oriented middleware that hides the complexity of OSGi and that extends OSGi/iPOJO in two ways:

- Developers describe an application by intention using a dedicated language as opposed to explicitly specifying the composition of components and bindings at design time;

- From the abstract description of the application architecture, a concrete architecture is computed and updated incrementally by resolving the dependencies between the components currently available in the execution environment. Due to the incremental and dynamic (just-in-time) construction and maintenance made possible by ApAM, applications are resilient to the opportunistic installation or disappearance of devices, sensors, actuators, and web services.

The project currently uses ApAM 0.0.8.

Fuchsia is an extensible tool for integrating and managing external communication protocols within OSGi. Most of modern applications require multiple sources of information such as web services, databases, and ubiquitous devices. The integration of these services becomes a major issue when dealing with a multitude of them. Until now, protocol integration can only be done on a case-per-case basis.

Fuchsia provides developers with a framework that allows them to deal with different protocols in a systematic way. Therefore, maintainability and testability are enhanced while reducing the learning curve for different protocols.

### ApAM

Figure 4. Main ApAM Concepts

A service is dynamic if it can appear and disappear, without notice, at runtime. A dynamic application is built from dynamic services. ApAM is an extensible service platform intended to support the execution of concurrent dynamic applications. ApAM aims at simplifying the design, development and execution of applications that must guarantee service continuity in a moving and unpredictable context, and in the presence of other, potentially competing, applications. ApAM includes the ApAM Core and a number of managers (ApAM plugins) that are designed to collectively support the execution of concurrent dynamic applications.

The ApAM machine is based on a component-service metamodel (called ApAM metamodel) that supports the definition, execution and management of component-based and service-based applications.

Figure 5. The component ApAM metamodel

- Specification. A specification is a first class object that defines a set of provided and required resources (in the Java sense). Compositions are designed and developed in terms of specifications. By analogy with the objet oriented computing, the Specification act as interface.

- Implementation. An implementation is related to one and only one specification through the “implements” relationship. An implementation is an executable entity (in Java) that implements all the resources defined in its associated specification, and that requires at least the resources required by its associated specification. In practice, an implementation must define a class that implements (in the Java sense) the interfaces of its specification.

- Instance. An instance is related to one and only one implementation through the “instanceOf” relationship. An instance is a runtime entity represented in the runtime platform (in our case, OSGi) as a set of Java objects where each object is an instance (in the Java sense) of its associated main class implementation. In the underlying runtime service platform (in our case, OSGi), an instance can be seen as a set of services, where one service corresponds to one resource of the specification. In ApAM, an instance is a Java object.

- Composite Implementation. A composite implementation, also called composite type, is a special case of implementation. As such, it implements a specification, but in contrast to an atomic implementation, a composite type does not define a main class. A composite type contains implementations and defines the rules that govern their composition and visibility.

The ApAM framework sources, binaries, documentation and tutorials are freely available at: https://github.com/AdeleResearchGroup/ApAM

### Fuchsia

Fuchsia is an extensible tool for integrating external communication protocols within an OSGi platform. It provides a common interface for creating and managing external connections.

Figure 6. Fuchsia Core Concepts

Most modern applications require multiple sources of information such as web services, databases or ubiquitous devices. The integration among those different services becomes a major issue when dealing with multitude of them. In ubiquitous systems, this kind of integration is even more important since the dynamicity and the multitude of protocols grows with the number of devices supported. As a result, developers are facing the challenge of mastering several protocols as well as their tools for debugging, dependency management, native calls access, access configuration, etc. This is where Fuchsia plays a key role, by providing the same way of dealing with different protocols, creating a single entry-point for all protocols supported. In addition, the developer can administrate and monitor all exchanges of an OSGi platform with external protocols through a unique control panel.

Fuchsia introduces 4 types of core concepts – declaration, importer and exporter, linker, and supports device/service discovery.

- Declaration. It is the metadata that describes a device or a service.

- Discovery is responsible for the detection of a device/service. Fuchsia creates a Declaration and adds it to the OSGi registry.

- Importer and Exporter. Whereas an Importer makes external protocols available to an OSGi platform, an Exporter makes OSGi services available from outside the OSGi platform.

- Linker. A linker connects a declaration to an importer or to an exporter provided that this importer/exporter satisfies the constraints specified in the declaration.

## Using the Project Source Code

### Useful Links (source code, documentation, bug reports)

The source code for the project is hosted on GitHub:

https://github.com/appsgate2015/appsgate

Documentation

Wiki:  https://github.com/appsgate2015/appsgate/wiki

Other documentation files (such as Server and Client companion) at :

https://github.com/appsgate2015/appsgate/tree/master/documentation

Issues and bug tracker:

https://github.com/appsgate2015/appsgate/issues

DailyTest reports are available on Cloudbees:

https://apam.ci.cloudbees.com/job/AppsGate%20Server/

### Requisite to build the project

Git is used for Source Code Management (SCM)

Java 6+ is used to build and run the project

Maven 3.2.x is the building tool

MongoDB must be available to execute the code locally

Any maven compliant IDE can be used (Netbeans, Eclipse, IntelliJ, …)

Chrome browser is preferable with AppsGateClient (with javascript enabled)

In addition to build the project (especially the first time), Internet Connexion is mandatory to download the dependencies.

### Building and running the server with command line tools

Assuming the previous tools have been successfully installed and are available on command line and mongoDB is started with default configuration values on localhost.

> git clone https://github.com/appsgate2015/appsgate.git

(download the latest source code)

> mvn clean install –Pcomplete

(builds the project, if build fail, please retry before further investigation)

> mvn javadoc:aggregate

(builds the javadoc inside appsgate/documentation/apidocs)

> cd distributions/complete

(goes to the preinstalled distribution)

> sh apam (for Linux)

or

> sh apamMacOSX (for MacOS)

or

> apam.bat (for Windows)

(launch the AppsGateServer)

Then open a web browser with the URL : http://localhost:8080/spok/index.html

### Project hierarchy

| Directory | Description & noticeable files |
| --- | --- |
| appsgate | Root directory See pom.xml to change: - generic properties - version of 3rd party dependencies - dependencies repositories - scm & profile configuration |
| distributions | Binary distributions and releases |
| felix.config.base | Empty distribution, with: - base configuration files - mandatory pre-installed binaries |
| light | Release that dynamically download and installs on-the-fly software component according to the context: - Available technologies and drivers - Availables web services (using ApAM OBRMan manager) |
| complete | Release that preinstall ALL possible software components and dependencies (used for minikit releases) |
| […] | specific and/or deprecated |
| documentation | Documentation directory |
| apidocs | Built by maven, contains all the javadocs of the project modules |
| appsgate | Base directory for all kind of documentation (including this one) |
| ard | Ard scripts and configuration files |
| domi_cube_node | Binaries, scripts and configuration files for domicube |
| sources | Source code for the project |
| appsgate-client | Sub-project containing the web client (HTML & Javascript) TODO : see Appsgate Client companion |
| appsgate-project | Source code for the server |
| android-demo-distrib | deprecated |
| appsgate-context | Components for Persistence and Context Management. Includes Specification and Implementation |
| appsgate-EUDE | Components for End-User Development Environment. Includes Specification and Implementation |
| appsgate-instance | Launcher component for AppsGate |
| communication-components | Components for Communication: - CHMIProxy - EHMIProxy - ClientCommunicationManager - TraceMan Includes Specification and Implementation |
| composite-distribution | ApAM Composite component that describe the whole appsgate-server architecture. |
| core-specifications | Specification components for all Core HMI Devices and Services Includes CoreObjectSpec |
| extended-specifications | Specification components for all Extended HMI Services |
| integrated-technologies | Implementations components for Core Objects, including - Core HMI Devices - Core HMI Services - Extended HMI Services - Technology Adapters |
| lib-utils | Third party source code that have been modified (repackaged or slight updates or bugfixes) to be used in the project. |
| test-components | Various components used for testing |
| CoreTest | Core Unit Tests for Appsgate |
| EUDTest | Tests for End-User Programs and Interpreter |
| exportMongo | Export Mongo Database to a CSV File ? |
| appsgate-pax-test | Integration Tests using Pax to run a whole OSGi container (allows to test composite ApAM and OSGi components) |
| […] | specific and/or deprecated |
| debugger | TODO: See reference document ? |
| stage | deprecated |

# Core HMI Middleware

As stated previously, the CHMI middleware bridges the gap between the external world and AppsGate. Its functional coverage includes:

- Technology Adapters integrates an heterogeneous set of sensors/actuators network protocols such as EnOcean, UPnP or Web services.

- Core Devices provides uniform representation and management of EnOcean/UPnP/etc. compliant physical devices as well as of third parties services, such as Google calendar available from the cloud.

- ApAM provides automatic support for dynamicity and resilience.

This chapter is divided in four parts. A small tutorial presents how to support a new Device Type and therefore introduce the main concepts of the middleware. The main uses cases of the middleware are presented along with sequence diagrams. Then comes API documentation for the Core Device and the Technology adapters. The complete list of currently supported Technologies and Core Devices are presented Annex B: Existing Core Services and Devices Specifications and Annex B: Existing Core Services and Devices Specifications respectively.

## Core HMI Middleware by the example: Adding a new Device Type

The Core HMI Middleware has been designed to cope with heterogeneous and unpredictable sets of devices and services in the home. In particular, it supports the arrival and departure of devices and services without requiring stopping or rebooting the system. This challenge is a canonical use case for the Core HMI middleware, as third parties may need to integrate new types of devices and services in an opportunistic manner.

### Process overview

Adding a new device (service) type DeviceType to the system is a three-step process:

- Step 1: Development of an ApAM compliant CoreDeviceType Specification to represent the new device type in the Core World.

- Step 2: Development of an ApAM compliant Communication Technology Adapter, responsible for a specific communication protocol between the physical devices of that type and the CHMI.

- Step 3: Development of a CoreDeviceType Implementation that holds the business code of the device type – it implements the interfaces of the CoreDeviceType Specification, and that binds the CoreDeviceType Specification to the Communication Technology Adapter.

To illustrate the process, we describe the step-by-step integration of a KNX®-based roller shutter actuator within an AppsGate Smart Home. In this running example, Roller Shutter is supposed to be a new device type and KNX a new network communication protocol within the Smart Home. Figure 6 shows the new components that need to be developed and integrated within the AppsGate Smart Home software architecture.

Figure 7. Integration of a KNX roller shutter actuator

### Step 1: Development of a Core Device Specification

As stated above, “Core World denotes the representation that the Core HMI Middleware has about the physical world and of the cloud digital world. This representation is expressed in terms of Core objects”. Therefore, for each device type DeviceType, there must be a CoreDeviceTypeSpecification that describes the type. In our example, this means that a CoreRollerShutterSpecification needs to be defined.

A CoreDeviceTypeSpecification is an ApAM component that describes a device type in an abstract manner by the way of a contract. This contract includes a set of:

- Java interfaces that describe the functions that can be called on the devices of that type.

- Properties that the devices of that type holds. For example, for sensor devices, these properties represent numeric observables about the real world, and for an actuator, the internal state of the actuator.

- Notification messages that the devices of that type can send. Notification messages are used to inform that observables from the real world or the internal state of the physical device have changed.

ApAM compiles and packages the CoreDeviceType Specification as an OSGi bundle. As a result, it can be used dynamically at runtime.

#### Recommendations

Note that, at this stage, Reusability should be a concern. Thus, before creating a new CoreDeviceTypeSpecification (e.g., a CoreRollerShutterSpecification) developers should pay attention to the set of existing specifications.

- If an existing specification provides the same business functions, this specification should be used. The original component should also serve as a guideline for the next steps.

- If the function is completely new, developers should design a component that can be reused and therefore abstracts the underlying technology (i.e. hides the details and specific extra functions relative to one single technology).

#### Design principles

Several design principles and constraints must be satisfied so that a CoreDeviceTypeSpecification can be retrieved and used by the CHMI Middleware.

Each business function that can be used remotely must be exposed as a programmatic Java method:

- These methods accept only simple parameters and return values (i.e. only primitive types and String are allowed).

- If a function is durative (typically, closing a roller shutter takes some time), this function should return immediately, returning void or a Boolean value to acknowledge that the processing of the command has started. A notification event will be sent asynchronously when the action of the command ends (e.g., when the shutter is closed).

A CoreDeviceTypeSpecification must provide the CoreObjectSpec interface:

- This interface provides methods that are shared by all Devices and Services.

- This Java interface is part of the API. Therefore nothing needs to be done at this level.

Device state changes must be provided as notific	ation messages:

- A notification message must implement the NotificationMsg interface (part of the API).

- A notification message must provide the id of the sender, the parameter that has changed (its name), the new value (if relevant) and the old value (if relevant).

- CoreNotificationMsg is provided as an helper class in the API and can be used directly as extra information is not used by CHMI.

The table below exemplifies the Core Specification for the device type RollerShutter.

#### Core RollerShutter Specification

| ApAM Specification Descriptor | Java Interfaces | Notification Messages |
| --- | --- | --- |
| <specification name="CoreRollerShutterSpec" interfaces="mypackage.CoreRollerShutterSpec, appsgate.spec.CoreObjectSpec" messages="appsgate.messages.NotificationMsg"> <!-- appsgate meta data --> <definition name="deviceId" type="string"/> <definition name="deviceType" type="string"/> <!-- Actuator specific metadata --> <definition name="rollerStatus" type="open,intermediate,close" /> <definition name="intermediateStatus" type="int" /> </specification> | package mypackage; public interface CoreRollerShutterSpec { /** * Send command to fully open the Shutter */ public void open(); /** * Send command to fully close the Shutter */ public void close(); /** * Set an intermediate level between * 0 (fully opened, min value) * 100 (fully closed, max value) */ public void setIntermediate(int value); } | This specification only sends CoreNotificationMsg. Examples : rollerStatus = open rollerStatus = close intermediateStatus = 23 |

### Step 2: Adding a Communication Technology Adapter

A communication technology adapter covers two main functions:

- Protocol-specific discovery of devices and therefore dynamic creation or destruction of the associated ApAM instances

- Proxy between the physical devices and their representation as Core Devices (not mandatory, but it may be convenient to have a single entry point for the technology specific protocol, especially for security concerns).

In order to create an ApAM instance, the communication technology adapter uses an ApAM Implementation of the Device type according to the technology and asks for the creation of a “new” object.

// 1° Search for an Implementation of the Device type (e.g., KNXRollerShutterImpl) Implementation apamImplem = CST.componentBroker.getImpl("KNXRollerShutterImpl"); // 2° Configure properties that may be used by the Device (e.g., a map key = value) Map<String,Object> configuration = new Hashtable<String,Object>(); configuration.put("property name", "property value"); //The following are mandatory properties configuration.put("deviceId", "a UNIQUE identifier"); configuration.put("deviceType", "the name of the category of devices"); // 3° Create an Instance of the Device type (a new Object is added to the registry) Instance apamInstance = apamImplem.createInstance(apamImplem, configuration); // 4° If necessary, get the physical service (a java interface defined by the CoreRollerShutter Specification) CoreRollerShutterSpec myShutter = (CoreRollerShutterSpec) apamInstance.getServiceObject();

Implementation of the adapter can be done in an ad-hoc fashion using specific libraries and API for the protocol or by using a protocol integration framework like Fuchsia.

Fuchsia provides developers with a framework that allows them to deal with different protocols in a systematic way and allows the deployment of new protocols dynamically. As an integration framework, Fuchsia facilitates the maintenance and testability of the integrated protocols. Fuchsia already embeds key protocols like KNX, UPnP, JSON-RPC, Bluetooth, etc.

Although Fuchsia already contains the protocols mentioned above, it is necessary to instantiate the proper components in the platform so that the protocol is effectively supported.

### Step 3: Development of a Core Device Type Implementation

An ApAM Implementation, CoreDeviceTypeImplementation, of a device type deviceType such as RollerShutter, according to a particular communication technology such as KNX, holds the business code of the device type and binds the CoreDeviceTypeSpecification to the appropriate Communication Technology Adapter. This CoreDeviceTypeImplementation must be fully compliant with the CHMI Middleware in order to be discovered and used in AppsGate.

#### CHMI Integration of the CoreDeviceTypeImplementation

The example, below, shows how to proceed according to the ApAM metamodel.

<implementation name="KNXRollerShutterImpl" classname="mypackage.KNXRollerShutterImpl" specification="CoreRollerShutterSpec" push="appsgate.messages.NotificationMsg"> <callback onInit="start" onRemove="stop"/> <!-- device kernel meta data --> <definition name="deviceId" field="appsgateObjectId" /> <definition name="userType" field="appsgateUserType" /> <!-- Actuator specific metadata --> <definition name="rollerStatus" field="currentRollerStatus" /> <definition name="intermediateStatus" field="currentIntermediateStatus" /> <relation field="KNXAdapter" specification="KNXAdapterSpec" /> </implementation>

A CoreDeviceTypeImplementation is an ApAM implementation component. As such, it has a classname that refers to the Java class that implements the interfaces specified in the associated ApAM Specification. Callback functions (e.g., start and stop) are called when an instance of the device type is created (e.g., onInit) or removed (e.g., onRemove) by the Communication Technology Adapter.

The properties of an ApAM Implementation Component, such as deviceId and rollerStatus, can be dynamically injected in Java fields of the main class. Changes are synchronized both ways (changes in the field within the object instance at runtime or changes of the property by ApAM).

An ApAM Implementation Component also defines relations to express dependencies with other ApAM components. Relations can also be dynamically injected in Java fields. For our roller shutter, the relation KNXAdapter allows to send commands to the physical KNX device using the corresponding KNX Communication Technology Adapter, KNXAdapterSpec.

The Java implementation is a straightforward process that maps commands from the CHMI to real commands sent to the Communication Technology Adapter and conversely, events received from the physical device to the corresponding notification messages. Several helper classes are provided to handle specific aspects of the HMI Middleware:

- AbstractObjectSpec is an abstract class that implements the generic CoreObjectSpec interface, the main class should extend this one and override the getDescription() method.

- CoreNotificationMsg implements NotificationMsg and can be used to create a message providing only the property’s name and value that have changed.

The getDescription() method must be overridden and should provide the current representation of the device as a JSON description.

@Override public JSONObject getDescription() throws JSONException { JSONObject descr = new JSONObject(); // mandatory appsgate properties descr.put("id", appsgateObjectId); descr.put("type", appsgateUserType); // Device specific properties descr.put("rollerStatus", currentRollerStatus); descr.put("rollerStatus", currentIntermediateStatus); return descr; }

## API Reference: Core HMI Uses Cases

Two mains actors use the Core HMI Middleware:

- The end-user wants to benefits from its Home device by controlling them with an AppsGate Client interface. Note: usually the EHMI Proxy acts on behalf of the Client.

- The Home Device itself that can be installed, configured or even breakdown (with the current version of the AppsGate Server, the installation and configuration issues are not part of this document).  Note: usually a Technology Adapter acts on behalf of the Device.

In the Core HMI Middleware, there is only minimal context information attached. Home Devices are only required to provide a unique objectId and a deviceType. This layer does not consider the location of the device, nor correlate with other perceptions of the same target or whatever.

Figure 8. Use Cases for the Core World

### A Home Device become Available or Unavailable

A Home Device may become available when the corresponding Technology Adapter enters in pairing mode, starts an explicit discovery or without any specific action (just plug and play).

When the device is discovered, a two steps process occurs.

Step One: ApAM (along with the Technology Adapter) is used to create the corresponding component (a Core Device Instance) in the Core HMI Middleware dynamically. Three properties must be provided during this step:

- ObjectId (also referred as DeviceId, ServiceId or id): is a unique identifier for the device. If the same device is lost and discovered again, the same id should be provided. A good practice is to concatenate the implementation name, the current version, and the MAC Address.

- DeviceType (also referred as ServiceType, UserType or type): is the category name of the Device. A good practice is to use the Specification Component or the main java interface name. Legacy devices types are defined as numbers. Please pay attention to reuse already existing device types (see Annex B: Existing Core Services and Devices Specifications).

- status (also referred as ObjectStatus): is the Connectivity state of the device, some values are predefined 0 if the device isn’t unreachable and 2 if the device is paired, configured and available, other codes may be defined accordingly to business functions (mostly failure codes). As most devices requires some specific configuration stuff. This status SOULD NOT be initially set to 2.

During this step, the Device is considered as unstable and therefore it is hidden to the Extended HMI Layer.

Step Two: is the configuration of the device. It may be an internal process, or uses the Technology Adapter.

When configuration is done and the Device is available, it must set its status property to 2 and send the corresponding Notification Message to be used by Extended HMI Layer.

When the device become unavailable. ApAM (along with the Technology Adapter) can destroy the corresponding instance. Or the status property may be changed by the instance itself. It is an implementation choice (the instance being able to monitor itself its availability or not).

The Technology Adapter itself can be created or removed at runtime using ApAM.

AppsGate Clients (and the EHMI Middleware) will be warned of this apparition or disappearance of device.

Figure 9. Sequence Diagrams for "an Home Device become Available" and “become Unavailable”

### The AppsGate client can get the Devices List or get one Device Status

The CHMI Proxy acts as a registry of the currently available Home Devices. Therefore its can provides the list to the AppsGate Client. But it is the duty of the Core Device itself to provide its current status (or the last known state of the physical apparatus). A status provided encompasses static properties of the device along with dynamically measured values.

Figure 10. Sequence Diagrams for “Appsgate Client get Devices List” and “get Device Status”

### The AppsGate client can send commands or retrieves events from the Device

The canonical use case of Core HMI is direct control of the Home Device, by sending commands (with an optional return value) or listening to events.

Figure 11. Sequence Diagrams for "AppsGate Client send Command" and "receive Event"

## API Reference: The CoreObjectSpec Component

CoreObjectSpec is the base artefact to implement or use any Home Device in the Core HMI Middleware.

Maven Project

As a Maven project, CoreObjectSpec provides a library of code containing the java interfaces along with a set of helper classes. It is therefore a development framework. To add it to your maven project set an appsgate.version property (currently 1.0.0) and the following maven dependency:

<dependency>

<groupId>appsgate.components</groupId>

<artifactId>CoreObjectSpec</artifactId>

<version>${appsgate.version}</version>

</dependency>

OSGi bundle

As an OSGi bundle, CoreObjectSpec is also an executable piece of code. It provides those API at runtime, acting as a single entry point between the heterogeneous Devices and Services.

ApAM Component

In order to be recognized by the CHMI Proxy, each Core Device (representation of a single Equipment of the home or WebService) must implements the ApAM CoreObjectSpec Specification along with several ApAM properties. This one represents the common ground to interact with a Device and allows dynamic discovery.

Figure 12. CoreObjectSpec Specification Component

The “main” class of the component MUST implements the java interface CoreObjectSpec (and only this main class).

The component SHOULD also provide specific java interfaces for its business functions. These functions must respect some constraints with the data types allowed for parameters or return values:

- The primitive data types: boolean, int, long, float, double

- Java classes from the java.lang package like String  or the object representations of the primitives types (Integer, Long, Float, Double). Beware that extending Class or Object will result in unpredictable behaviour, they should be considered as unsupported

- JSONObject or JSONArray fom org.json package.

Classes of the component MAY send messages implementing the NotificationMsg relative to business events from the device.

### Core Object properties

Any Core Device provide a minimal and mandatory subset of ApAM properties:

| Name | Type | Description |
| --- | --- | --- |
| id | String | Reference for the device, it must be unique on the platform |
| type | String | Associated to each class of Device (see those existing in Annex B: Existing Core Services and Devices Specifications) |
| status | Number | Connectivity state of the device, some values are predefined 0 if the device isn’t unreachable and 2 if the device is paired and available, other codes may be defined accordingly to business functions (mostly failure codes) |

The Core Device CAN defines its own properties representing the internal state or configuration of the device. Device property values can dynamically change as times passes by, but refresh rate is a matter of concern. A state value changing several times in less than one second SHOULD not be represented as a property or performance issues might appear. Consider using a getter method or a value update event.

ApAM provides the following data type for properties (see ApAM documentation about property definition, setting, behaviour and injection):

- Basic types are int (same as integer), string, float and boolean.

- An enumeration is a comma-separated list of string. Values must not contain comma. White spaces are ignored around the commas. A definition can include a default value, as for “bedroom” above.

- A type can also define a set of the previous types.

### CoreObjectSpec generic class interface

The ApAM descriptor for any implementation component provides a classname acting as an entry point for the component. The corresponding class MUST implements the java interface CoreObjectSpec.

This interface only provides getter methods (with no parameters) as follows:

| Name | Return | Description |
| --- | --- | --- |
| getAbstractObjectId() | String | Getter for ApAM property: returns the property id, acting as unique identifier for the Device. The id representing one particular instance of device |
| getUserType() | String | Getter for ApAM property: returns the property type, the category of Device. All instance of this class of Device (even using other Technology Adapters) will share the same number |
| getObjectStatus() | Number | Getter for ApAM property: returns the property status, the number representing the connectivity state of the device. The status may change during time, this value can be used to diagnose failure |
| getCoreType() | CORE TYPE | Return the meta category of CoreObject, can be an equipment (DEVICE) a web service (SERVICE) a communication technology adapter (ADAPTER) or an extended service (EXTENDED) |
| getDescription() | JSONObject | Returns current property values for the device, it include the preceding mandatory ApAM properties, configuration values, state variables relative to the category of device, … (see 4.3.4) |
| getBehaviorDescription() | JSONObject | Returns a grammar for the device, relative to its category, defines its states, events, commands (the device expressed as a finite-state machine). This one is used for End-User Programming (see 4.3.5) |

### NotificationMsg generic message interface

The ApAM descriptor provides a list of push messages (events) generated by the component. In order to be handled by the CHMI Proxy, those messages MUST implements the java interface NotificationMsg.

This interface only provides getter methods (with no parameters) as follows:

| Name | Return | Description |
| --- | --- | --- |
| getSource() | String | Returns the object id of the device that has triggered the message |
| getVarName() | String | Getter for the name of the property that has changed |
| getNewValue() | String | Getter for the new value of this property (after property change) |
| getOldValue() | String | (Optional) Getter of the old value of the property (before property change) or null if unused |
| JSONize() | JSONObject | Returns A JSON representation of the event (see 4.3.6) |

### Core Object Description (JSON)

The JSON description provides all properties that can be used to understand the Home Device current status.

It is a set of key/values attributes. Values can be Strings, Numbers or Booleans, but also Arrays or even Embedded JSON Objects.

It includes the mandatory properties (ObjectId, UserType and ObjectStatus).

It includes configuration values, state variables and all kind of business properties specific to the UserType. Part of these properties may be computed at real time.

For example with the roller shutter, getDescription() could returns:

{

id :"KNXRollerShutterImpl-123" ,

type :"CoreRollerShutter" ,

status : 2,

rollerStatus :"intermediate" ,

intermediateStatus : 75

}

### Core Object Behavior (JSON)

The behaviour is a static JSON grammar used to describes the semantics of its actions, properties, states and events. It is relative to a category of device but might be overridden for a particular implementation.

Along with the actions, events, properties and state variables, an Home Device can be described as a Finite-State-Machine, with events triggered when the automaton enters or leaves a state, and actions (methods call) called for state changes.

This behaviour description is used by the End-User Development Environment and the debugger tools.

For example with the roller shutter, getBehavior() could returns:

{

friendlyName :"Roller Shutter" ,

typename :"CoreRollerShutter" ,

traceDesc : ["state" ,"rollerStatus" ,"intermediateStatus"],

properties : ["rollerStatus" ,"intermediateStatus"],

states: [

{

name: "isOpen",

stateName : "getRollerStatus",

stateValue : "open",

setter: {

type: "action",

methodName: "open"

},

startEvent: {

type: "event",

eventName: "rollerStatus",

eventValue: "isOpen"

}

},

[...]

],

commands: {

open : {},

close : {},

setIntermediate : { params : [{name : "value"}]},

}

}

### Notification Message Description (JSON)

Each time a property value change, the Home Device SHOULD send a message describing this value change. Other messages might also be sent upon entering or leaving state (see 4.3.5).

Those message MUST contain one single property (with a single varName, and a single value). Each value type is String.

For example with the roller shutter, JSONize() of a message could returns:

{

objectId :"KNXRollerShutterImpl-123" ,

varName :"rollerStatus" ,

value : "open" ,

oldValue : "intermediate"

}

### Helper Classes

Along with the interfaces, several helper classes are provided.

Figure 13. Packages from the CoreObjectSpec

CoreNotificationMsg

This concrete class is the base implementation for NotificationMsg. It provides a simple and reliable implementation of the interface. It CAN and SHOULD be used unaltered.

Overriding this class or providing another implementation is useless, as extra features won’t be used by the Core nor the Extended HMI Middleware.

CoreObjectBehavior

This abstract class implements CoreObjectSpec. It provides a simple implementation of getBehaviorDescription(). The developer must provide a file grammar.json (see 4.3.5) located in the same package as the implementation.

This class CAN and MUST be overridden as it does not provides the methods expected of CoreObjectSpec.

AbstractObjectSpec

This abstract class implements CoreObjectSpec. It provides a simple implementation of all functions. It is declared abstract because it should serve as an example.

This class MUST NOT be overridden, because inherited fields may cause problems with dynamic code injection.

StateDescription

This concrete class is a wrapper for behaviour description providing some convenient getters and setters.

### Technology Adapters

A Technology Adapter should be developed in the same way as any Home Device except it is not designed to be used in End-User Programming. Therefore it SHOULD NOT provide behaviour description and the corresponding CORE_TYPE is ADAPTER.

# Extended HMI Middleware

The purpose of the Core HMI Middleware is to provide abstractions for Home Devices and Services. Each of the Core Objects is independent of the other Devices or the current context.

The Extended HMI Middleware is by design application dependent. It correlates or aggregate data from multiple Devices, extract or provides Context information. It provides an extensible set of software components designed for facilitating the development of smart environments.

## Extended HMI Uses Cases

The only main actor that uses the Extended HMI Middleware is the AppsGate Client (and therefore the End-User).

It provides two canonical uses cases, to send a command or receive an event. It then delegates the function to a dedicated extended service.

In the following, we present specific uses cases that are related to Context Management and End-User Programming. The corresponding software components are provided along with the platform.

Figure 14. Extended HMI Middleware Use Cases

Context Management is dedicated to several Extended Services known as Context Managers. For the moment only a few are provided (for set-based location, naming of devices, and trace history), they mostly provides persistency means for extra semantics of the Core Devices. The existing managers with the platform can be overridden or new Context Managers can be designed.

Figure 15. Typical Sequence Diagrams for Context Management

## API Reference: Context Management

As stated previously Context Management mostly acts as a persistency tools to store the configuration of the connected Home and its history. But each component can be easily overridden to add perception and identification layers.

It is divided according to semantic domains:

- DependencyManager dynamically computes dependencies between Devices, Places and End-User Programs.

- PropertyHistoryManager stores the history of ApAM property changes (it is designed to log every changes in Devices and Services status, including sensors values or actions).

- PlaceManager is a Set-based location manager. Each place can contain several Devices or other Places.

- DevicePropertiesTable stores user knowledge about Devices and Services such as a friendly name.

- UserBase is not used for the moment. It is designed to grant privilege for using the Devices and Services. It also associates particular DeviceProperties to particular users.

- MongoDBConfiguration is an helper component to store or retrieve any kind of Information from the DataBase.

Figure 16. Context Management Components

### API Reference: MongoDBConfiguration

The MongoDBConfiguration projects handle the Database Management using MongoDB.

A MongoDB Server  (see http://docs.mongodb.org) can run in the networks and provides connections (in our case the MongoDBConfiguration is the only client).

A record in MongoDB is a document, which is a data structure composed of field and value pairs. MongoDB documents are similar to JSON objects. The values of fields may include other documents, arrays, and arrays of documents.

MongoDB stores documents in collections. Collections are analogous to tables in relational databases. Unlike a table, however, a collection does not require its documents to have the same schema.

Figure 17. Packages from MongoDBConfiguration

MongoDBConfigFactory

This concrete class is the entry point for the component. It manages the configuration properties to connect to Mongo Server.

At start-up, it reads the conf/config.properties to retrieves a specific configuration URL (it can be a file or an http server):

fr.imag.adele.apam.managers.configuration= ...

MongoDBConfiguration:file:./conf/defaultMongoDBConfig.cfg

...

This configuration file should provide the following basic properties.

#Default property file to configure the Mongo DB

DBHost=127.0.0.1

DBPort=27017

DBName=defaultDB

DBTimeout=3000

dropCollectionsOnStart=false

These properties can be changed at runtime, using the ApAM component properties or the method public void setFactoryParameters(Properties props).

| Name | Type | Description |
| --- | --- | --- |
| DBHost | String | Host name or IP address for mongo server (127.0.0.1 – localhost by default) |
| DBPort | int | IP Port for the mongo server (27017 by default) |
| DBTimeout | int | Timeout (in ms) for mongo server connection |

MongoClientConfigurationThread

This concrete class is a Thread that tries to connects or reconnects to Mongo Server using the configuration properties.

When the creation is successful it creates the MongoDBConfiguration component.

It also monitors if the connection keeps alive as times passes by and destroy the MongoDBConfiguration component if connection fall.

MongoDBConfiguration

This concrete class is also an ApAM component and a service that can appear or disappear dynamically upon the current connection status. Each ApAM component that is client of this component can react accordingly.

It provides several convenient methods to use the Database without dependencies to Mongo business wrappers.

| Type | Method and Description |
| --- | --- |
| static boolean | checkMongoClient(com.mongodb.MongoClient mongoClient) |
| boolean | dropCollection(String dbName, String collectionName) Try to remove an existing Collection in the specified dataBase (if dbName or collection unknown does nothing) |
| boolean | dropDB(String dbName) Try to remove an existing DataBase (if dbName unknown does nothing) |
| Set<String> | getCollections(String dbName) Get all the collections names (wrapping call to mongo API) |
| Set<String> | getDatabases() Get all the collections names (wrapping call to mongo API) |
| com.mongodb.DB | getDB(String dbName) |
| DBHelper | getDBHelper(String dbName, String collectionName) Gets an helper class that wraps simple operation with MongoDB collection (mostly insert and remove operations) |

DBHelper

This helper class provides methods to use a MongoDBCollection.

| Type | Method and Description |
| --- | --- |
| boolean | dropAndInsertJSONArray(org.json.JSONArray array) |
| boolean | dropAndInsertSetSimpleObject(Set<Object> set) |
| boolean | dropAndInsertSetString(Set<String> set) |
| Set<JSONObject> | getJSONEntries() |
| Set<Object> | getSimpleObjectEntries() |
| boolean | insertJSON(org.json.JSONObject obj) Insert a JSON object The object will get automatically an _id (if the key is not the id) If already existing in the DB, does nothing |
| boolean | insertSimpleObject(Object obj) Insert a Simple object (such as a String) The object will be the _id If already existing in the DB, does nothing |
| boolean | insertSimpleObject(String key, Object obj) Insert a Simple object (such as a String) The object will get automatically an _id (if the key is not the id) If already existing in the DB, does nothing |
| boolean | remove(String entryID) |

### Device Properties Manager Service

The Device Properties Manager is designed to hold properties about Home Devices that might be configured by end-users, such as a friendly name. Each end user might configure his own properties.

The current implementation only manages the name actually.

This service is also a helper class that manage in a single registry the relationship between devices types, device id and the associated grammar (the behaviour). All these relationships are dynamically discovered using the Core HMI Middleware.

This service provides an interface for the service: DevicePropertiesTableSpec and dedicated NotificationMsg to relates changes.

DevicePropertiesTableSpec

| Modifier and Type | Method and Description |
| --- | --- |
| boolean | addGrammarForDevice(String deviceId, String deviceType, GrammarDescription grammar) Add a grammar for a device type |
| void | addName(String objectId, String usrId, String newName) Add a new name to the hash map |
| void | deleteName(String objectId, String usrId) Delete an object name in the ash map |
| GrammarDescription | getGrammarFromDevice(String deviceId) Get the grammar associate to a device type |
| GrammarDescription | getGrammarFromType(String deviceType) Get the grammar associate to a device type |
| String | getName(String objectId, String usrId) Get the name give to a device by a specified user |
| String | getType(String deviceId) |
| boolean | removeGrammarForDeviceType(String deviceType) Remove a grammar associate to a device type |
| void | setType(String deviceId, String type) set a type of a device |

### Context History Manager Service

TODO

Please refer to the online documentation (appsgate/apidocs)

### Places Manager Service

TODO

Please refer to the online documentation (appsgate/apidocs)

### Users Manager Service

TODO

Please refer to the online documentation (appsgate/apidocs)

## End-User Programming

TODO

Refers to another document ?

## Other provided Extended Services

TODO

Please refer to the online documentation (appsgate/apidocs)

## API: Guidelines for Extended Services Integration

TODO

There are no special guidelines for the Extended Services Integration at the moment.

1° Reusable services (such as aggregation/fusion/filtering of other CoreDevices) should implement CoreObjectSpec specification: see 4.3.

2° Other extended services with close integration with End User Development, Context or Debugger, require code introspection of the corresponding dependencies.

# Communication between CHMI, EHMI and AppsGate Clients

## Communication Architecture

The AppsGate Server is divided in two layers:

In the Core World, Core Objects might interact among each other (at least this is the case between a Technology Adapter and the related Devices and Services).

In the Extended World the services are likely to provide added value with the combination of several Extended Services.

But, in order to preserve consistency and causal relation between events, actions and device states components in the Core World SHOULD NOT directly interact with components in the Extended World.

In addition, they SHOULD NOT interact directly with the AppsGate Clients. The AppsGate Client MUST have one single entry point with the Server.

All this emphasizes the necessity of a third section on the Functional Architecture: The Communication part.

Figure 18. Interaction between the Communication components

In order to illustrate the communication flow, we take one example with the End User starting a simple program that turns every lamp located in the kitchen off.

Figure 19. Sequence Diagram turning off a Lamp

## CHMI Proxy Service

As an entry point fore the Core HMI Middleware, this service is really a Proxy and therefore provides three functions:

- A dynamic registry of the Core Objects

- Routing messages (from) and method calls (to) specific Core Objects

- It translate Programmatic Java interfaces from/to Declarative JSON messages

Registry

Home Devices and Service and therefore the corresponding Core Objects may appear in an unpredictable manner and disappear with no warning.

CHMI Proxy monitors those Core Objects, checks their availability status and maintains an up-to-date registry. It sends CoreUpdate events when the registry is updated.

Il also provides upon demand a full JSON Description of these Core Objects (including business state variables and real-time computed properties).

Routing

As a registry, the CHMI Proxy is also able to address each Core Object using its objectId. It can also browse and broadcast along the userType.

It can route a Generic Command from outside the Core World (from EHMI Proxy) to the real Core Object able to execute this command. It also route back the command response if appropriate.

It also collect and convey the Notification Messages from the Core Objects to external CoreEventListener (to EHMI Proxy).

Translation

The Core Object are by design made to be used by AppsGate clients. Those clients are mostly web-based, the concerns and the development pattern are slightly different from those used in a server application, including very loose coupling, asynchronous communication, and declarative programming.

The common ground to interact with CoreObject, sending a description, asking for a command, notification of an event are JSON Message that will be easily handled by the client side.

The CHMI is responsible for making this translation between JSON and Java both ways.

Figure 20. Architecture of CHMI

Figure 21. CHMIProxySpec Project

### CHMIProxySpec service interface

| Modifier and Type | Method and Description |
| --- | --- |
| boolean | CoreEventsSubscribe(CoreEventsListener coreEventsListener) Subscribe to core events notification |
| boolean | CoreEventsUnsubscribe(CoreEventsListener coreEventsListener) Disable subscription for core events notification |
| boolean | CoreUpdatesSubscribe(CoreUpdatesListener coreUpdatesListener) Subscribe to core updates (add/remove object) |
| boolean | CoreUpdatesUnsubscribe(CoreUpdatesListener coreUpdatesListener) Disable subscription for core updates notification |
| GenericCommand | executeCommand(int clientId, String objectId, String methodName, JSONArray jsonArgs, String callId, AsynchronousCommandResponseListener listener) Execute command from outside to a specific device |
| String | getCoreClockObjectId() Get the identifier of the core clock |
| CoreObjectSpec | getCoreDevice(String objectId) Get the core object from its identifier |
| long | getCurrentTimeInMillis() Get the current system time in milliseconds |
| JSONObject | getDeviceBehaviorFromType(String type) Get the grammar behavior of a particular device type |
| JSONObject | getDeviceDescription(String objectId) Get the device description |
| JSONArray | getDevicesDescription() Get all the devices description as JSONArray |
| JSONArray | getDevicesDescriptionFromType(String type) Get all the device that fit the type parameter |
| JSONArray | getDevicesId() Get all the devices description as JSONArray |
| JSONArray | getDevicesIdFromType(String type) Get all the device that fit the type parameter |
| double | getTimeFlowRate() Get the current time flow rate |
| int | registerTimeAlarm(Calendar calendar, String message) Register a time alarm |
| void | restart() restart the system (Restart the system bundle from OSGi) |
| void | shutdown() Shutdown the system (Shutdown the OSGi distribution) |
| void | unregisterTimeAlarm(Integer alarmId) Unregister a alarm from its identifier |

### Generic Command

The CHMI Proxy is able to manage four categories of JSON messages:

- The Core Object Description described in 4.3.4, providing a set of properties representing the current device status

- The Core Object Behaviour described in 4.3.5, providing the device grammar

- The NotificationMsg described in 4.3.6 only provide the property that changed

- A GenericCommand to be executed

The Generic Command is also a key/value JSON message with several properties relative to the Client Communication Manager.

| Name | Type | Description |
| --- | --- | --- |
| clientId | int | (required) Unique identifier for the AppsGate Client that requested the Command. |
| objectId | String | (required) Unique Identifier for the Core Object as seen in 4.3 |
| methodName | String | (required) Method name that must match exactly the one provided for the Device Category |
| jsonArgs | JSONArray | (required but might be empty) A list of the methods arguments as key/value JSON , each one containing the data type and the data value |
| callId | String | (optional) identifier for the call in order to route the response |
| listener | x | (optional) The java object that should receive asynchronously the command response (should be EHMIProxy or null) |

## Client Communication Manager Service

This small service is responsible of communication with external applications that does not run within the AppsGate server. Typically, it handles communication with the AppsGate Clients using WebSocket.

Figure 22. Client Communication Manager Project

WebSocketServer

This concrete class opens a Web Socket server. Port number should be configured using OSGi property org.lig.appsgate.websocket.port (if not provided, default value is 8087).

SendWebsocketsService

This interface defines how to send messages to connected applications (AppsGate clients). Those messages should be provided by EHMIProxy and formatted as valid JSON Strings.

- A message can be broadcast to all connected clients (for instance a state change of a Core Object).

- A message can be sent to one clientId only.

- A message can be sent as the asynchronous response of a request.

ListenerService

This interface defines how a listener can receive commands from the connected applications (AppsGate clients). EHMIProxy should be the only listener.

Nevertheless, any CommandListener can subscribe to the ListenerService, being registered with a TARGET name. The client with the request specifies the TARGET he wants to address.

Due to legacy and history reasons several Listener have been defined so far:

- EHMI: request are routed to EHMIProxy, this should be the only target

- CHMI: request are routed to CHMIProxy, now obsolete and unused

- ENOCEAN: request are routed to UbikitAdapter, now obsolete and replaced with the EnOcean Adapter as Core Device

- WATTECO: request are routed to Watteco Adapter, now obsolete and unused (technology is no more available)

- PHILIPSHUE : request are routed to the Philips Hue Adapter. That should be replaced soon by a HUE Adapter as Core Device

- UPNP: request are routed to the UPnP Adapter. This is almost unused

## EHMI Proxy Service

The EHMI Proxy is the key of the AppsGate Server. It manages communication between the set of Extended Services, the CHMI Proxy and the Communication Manager.

To sum up, it handle in some way every communication from/to:

- the Extended HMI Middleware (its main purpose)

- the Core HMI Middleware

- the AppsGate Clients

Therefore it can intercept communication, add extra information and delegate methods.

Work-in-progress

For the moment, EHMIProxy has explicit dependencies with each Context Manager, with EUDE Interpreter, with Dependency Manager, Trace Manager….

If the CHMIProxy is generic using abstract CoreObjects, and if most of the Extended Services implements Core Object, some uses cases of the Extended HMI remains hardcoded as method call of EHMIProxySpec interface. The EHMI being itself some kind of Core Object.

EHMIProxySpec

| Modifier and Type | Method and Description |
| --- | --- |
| boolean | addClientConnexion(CommandListener cmdListener, String name, int port) Add a new client connexion. |
| void | addCoreListener(CoreListener coreListener) This method allow the caller to add a specific coreListener to follow core components state change. |
| boolean | addProgram(org.json.JSONObject jsonProgram) Deploy a new end user program in AppsGate system |
| boolean | callProgram(String programId) Run a deployed end user program |
| boolean | checkProgramIdScheduled(String programId) Check if a particular program Id is scheduled in the future (on start or on ending) |
| JSONArray | checkProgramsScheduled() Check programs that are scheduled in the future (on start or on ending) |
| void | deleteCoreListener(CoreListener coreListener) This method allow the caller to unsubscribe itself from core components state change. |
| GenericCommand | executeRemoteCommand(String objIdentifier, String method, org.json.JSONArray args) Get a runnable object that can execute command from a remote device manager asynchronously |
| String | getCoreObjectPlaceId(String objId) Get the place identifier of a core object |
| long | getCurrentTimeInMillis() Get the current system time from the local EHMI clock |
| JSONObject | getDevice(String deviceId) Get device details |
| JSONArray | getDevices() Get all the devices description |
| JSONArray | getDevices(String type) Get all the devices of a specify user type |
| ArrayList<String> | getDevicesInSpaces(ArrayList<String> typeList, ArrayList<String> spaces) Return the devices of a list of type presents in the places |
| StateDescription | getEventsFromState(String objectId, String stateName) Get the state representation for a device type in the grammar |
| GrammarDescription | getGrammarFromDevice(String deviceId) Get the grammar associated to a device |
| GrammarDescription | getGrammarFromType(String deviceType) Get the grammar associated to a device type |
| JSONObject | getGraph(Boolean buildGraph) |
| JSONArray | getPlaces() |
| SpokObject | getProgram(String programid) Return a list of |
| SpokObject | getProgramDependencies(String programid) Return a list of |
| JSONArray | getPrograms() Get the list of current deployed programs |
| JSONObject | getTraceManStatus() Get trace man status |
| boolean | isProgramActive(String programId) Check if a program is active or not |
| void | moveDevice(String objId, String srcPlaceId, String destPlaceId) Move a device in a specified place |
| boolean | removeClientConnexion(String name) Remove an existing client connexion |
| boolean | removeProgram(String programId) Remove a currently deployed program. |
| boolean | removeProperty(String placeId, String key) Remove a property from a specified place |
| void | restart() restart the AppsGate system (Restart the system bundle from OSGi) |
| void | scheduleProgram(String eventName, String programId, boolean startOnBegin, boolean stopOnEnd) Try to schedule the start or stop of a program The Event is created just one hour before current Time, and last for 30 minutes (it up to the end user to modify this event according to its need) |
| void | sendFromConnection(String name, int clientId, String msg) Send message to on client throught a dedicated connection |
| void | sendFromConnection(String name, String msg) Send message to all clients behind a dedicated connection |
| void | shutdown() Shutdown the AppsGate system (Shutdown the OSGi distribution) |
| int | startDebugger() Start the debugger on a new client connexion |
| boolean | stopDebugger() Close the debugger client connexion |
| boolean | stopProgram(String programId) Stop a deployed program execution |
| boolean | updateProgram(org.json.JSONObject jsonProgram) Update an existing program |

Recommendations for future work and refactoring of EHMIProxy

A lot of work has been undertaken on EHMI Proxy to transform Extended Services in Core Object, thus providing abstraction. But the Core Object pattern is not efficient for service aggregation.

One way is to build an ontology of expected behaviour. For instance, Context Managers might be a generic service concept providing extra-information upon Core Object Id or filtering Core Object Selectors.

Other mechanisms should be implemented as a generic publish subscribe of an extended service, interested in what happen with a category of message, a category of Device. The interesting case of End-User Development is a good example.

# Annex A: ApAM components Application Architecture

TODO

Diagram of the specs components and the Composites

# Annex B: Existing Core Services and Devices Specifications

## Existing Core Service/Device Type Codes

Important note: The code values being number are a legacy issue. A code is of type: String.  And the current recommendation for new DEVICE, SERVICE, ADAPTER, EXTENDED is to use the name of the specification component.

| Value | Friendly name | Category | Status |
| --- | --- | --- | --- |
| -1 | Undefined | DEVICE | Supported |
| 0 | Temperature | DEVICE | Supported |
| 1 | Illumination | DEVICE | Supported |
| 2 | Switch | DEVICE | Supported |
| 3 | Contact | DEVICE | Supported |
| 4 | Key card switch | DEVICE | Supported |
| 5 | Occupancy | DEVICE | Supported |
| 6 | Smart plug | DEVICE | Supported |
| 7 | Color light bulb | DEVICE | Supported |
| 8 | On/off actuator | DEVICE | Supported |
| 9 | Co2 | DEVICE | Supported |
| 31 | Media player | DEVICE | Supported |
| 36 | Media browser | DEVICE | Supported |
| 124 | TV | DEVICE | Supported |
| 200 | Mobile device | DEVICE | Supported |
| 210 | DomiCube | DEVICE | Supported |
| CoreFairyLightsSpec | Fairy Lights Actuator | DEVICE | Supported |
| MobileDevice | MobileDevice | DEVICE | Supported |
| 21 | System clock | SERVICE | Supported |
| 101 | Google calendar | SERVICE | Supported |
| 102 | mail | SERVICE | Supported |
| 103 | Weather | SERVICE | Supported |
| 104 | TextToSpeech | SERVICE | Supported |
| CoreEnergyMonitoringGroup | EnergyMonitoringGroup | SERVICE | Supported |
| EnergyMonitoringAdapter | EnergyMonitoringAdapter | ADAPTER | Supported |
| WeatherAdapterSpec | Yahoo Weather Adapter | ADAPTER | Supported |
| FairyLightsAdapterSpec | FairyLights Adapter | ADAPTER | Supported |
| MobileDeviceAdapter | MobileDeviceAdapter | ADAPTER | Supported |
| EUDE_InterpreterSpec | EUDE Interpreter | EXTENDED | Supported |
| PlaceManagerSpec | Place Manager | EXTENDED | Supported |
| DevicePropertiesTableSpec | Device Props Manager | EXTENDED | Supported |
| UserBaseSpec | User Base Manager | EXTENDED | Supported |

## CoreSmartPlugSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreSwitchSensorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreTempSensorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreCO2SensorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreContactSensorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreKeyCardSensorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreLumSensorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreOccupancySpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreOnOffActuatorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreColorLightBulbSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreMailSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreMediaBrowserSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreMediaPlayerSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreMobileDeviceSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreShutterSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreDomiCubeSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreTVSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreBadgeDoorSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreCalendarSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

## CoreClockSpec

TODO

Please refer to the online documentation (appsgate/apidocs)

# Annex C: Currently Supported Technology Adapters

## Clock and Alarm Service

Specific java implementation for appsgate

TODO

Please refer to the online documentation (appsgate/apidocs)

## EnOcean Devices using Ubikit Adapter

Low-consumption and energy harvesting protocol

Drivers and lower technology software provided by partner of the AppsGate project: Immotronic

TODO

Please refer to the online documentation (appsgate/apidocs)

## Philips HUE Lamps

Philips API and Fuchsia Integration

TODO

Please refer to the online documentation (appsgate/apidocs)

## UPnP Media players and browsers

Using UPnP base driver

TODO

Please refer to the online documentation (appsgate/apidocs)

## Domicube

Specific device developed by project Amiqual4Home, with Fuchsia integration

TODO

Please refer to the online documentation (appsgate/apidocs) and the wiki

## Weather Web Service

Using Yahoo Web Service

TODO

Please refer to the online documentation (appsgate/apidocs)

## Mail Web Service

Using SMTP/IMAP service adapter (mostly using gmail addresses)

TODO

Please refer to the online documentation (appsgate/apidocs)

## Calendar Web Service

Using Google calendar service

TODO

Please refer to the online documentation (appsgate/apidocs)

## Pace TV Service

Integration of software provided by partner of the project : Pace

TODO

Please refer to the online documentation (appsgate/apidocs)

## ARD alarm service

Integration of software provided by partner of the project : ARD

TODO

Please refer to the online documentation (appsgate/apidocs)

## KNX Devices

Currently in development, using Fuchsia

## Watteco Devices

Not supported anymore